SAMPLE LOG SHEET – This sheet should be modified to suit the type of pool (see notes on reverse)

<Insert Name of centre> <Insert name of pool>


Date _______________

Pool Water Testing

	
	<Insert POR range for this pool>
	<Insert POR range for this pool>
	Break point 0.0mg/L at the first test of the day
	pH range 7.0-7.8
	Total Alkalinity range 80-200mg/L 
	Cal. Hard

mg/L 
	Corrective actions taken
	Name of tester
	Signature of tester

	Time testing Due
	Time testing conducted
	Temp 

oC
	Total Chlorine mg/L 

DPD 1+3 (a)
	Free Chlorine mg/L

DPD 1

 (b)
	Combined Chlorine mg/L

Total-Free 

(a-b) 
	pH
	Total Alkalinity
	
	
	
	

	6am
	
	
	
	
	
	
	
	
	
	
	

	9am
	
	
	
	
	
	
	
	
	
	
	

	12pm
	
	
	
	
	
	
	
	
	
	
	

	3pm
	
	
	
	
	
	
	
	
	
	
	

	6pm
	
	
	
	
	
	
	
	
	
	
	

	9pm
	
	
	
	
	
	
	
	
	
	
	

	Daily Ave.
	
	
	[image: image1.png]



	
	
	
	
	
	
	
	


Cyanurate acid concentration (weekly measurement)_____________  Date ___________
Water Balance 







Using the water balance chart on reverse, calculate the Langelier Saturation Index (LSI). The ideal LSI is 0.2 and the range is –0.5 to 0.5.  

[image: image2.png]



____   +  _____  +  ____  +  ____  -  12.1  =  _______
Calculated by (signature) _____________________________

  pH            TF          CF         AF                        LSI   

Daily Maintenance Log Detail what was undertaken and at what time of the day. 

	Maintenance area
	Maintenance undertaken
	Time of maintenance
	Signature 

	
	
	
	

	
	
	
	

	
	
	
	


Review by manager Data and corrective action to be reviewed by facility manager daily
	Further action required or taken or other comments
	Name of Manager
	Signature of manager
	Date

	
	
	
	


Water Balance Chart

	Temperature (C°)
	Temperature Factor
	Calcium (Hardness)
	Calcium Hardness Factor
	Total Alkalinity
	Alkalinity Factor

	0
	0.0
	5
	0.3
	5
	0.7

	3
	0.1
	25
	1.0
	25
	1.4

	8
	0.2
	50
	1.3
	50
	1.7

	12
	0.3
	75
	1.5
	75
	1.9

	16
	0.4
	100
	1.6
	100
	2.0

	19
	0.5
	150
	1.8
	150
	2.2

	24
	0.6
	200 
	1.9
	200
	2.3

	29
	0.7
	300
	2.1
	300
	2.5

	34
	0.8
	400
	2.2
	400
	2.6

	40
	0.9
	800
	2.5
	800
	2.9

	53
	1.0
	1000
	2.6
	1000
	3.0


[image: image3.png]



Combined chlorine must not exceed 1 mg/L and should not be more the half the free chlorine





LSI = pH + TF + CF + AF - 12.1�where:�TF = Temperature factor�CF = Calcium Hardness factor�AF = Alkalinity Factor








Notes for modifying this log sheet for your pool





�	Use a separate sheet for each pool for each day.


�	Print on different coloured paper for each different pool.


�	Change the Health Prescribed Operating Requirement (POR) levels so they are appropriate for the type of pool. This depends on if the pool is indoor or outdoor, the temperature and the type of disinfection used. See Public Health Regulation 2012, Schedule 1. 


�	If used for a bromine pool, the breakpoint is not really important, and DPD 1 measures bromine.


�	If used for an ozone pool, insert a space to record when a check is done and the results.


�	If an automatic controller is used, and the results are input, then there needs to be a place to enter a manual reading to compare with the automatic.











[image: image4.png]


[image: image5.png]


[image: image6.png]


