POV INSPECTION CHECKLIST

ITEM	WHAT TO CHECK

LOOK FOR KNOWN DEFICIENCIES

CHECK OFF

TIRES

Condition

NOTE: No mixing of radial tires and bias tires.

Spare tire

Tread depth, wear, weathering, evenly seated, bulges, imbedded objects, cuts, breaks. At least one mm of tread over entire traction surface.

(Using a penny, place it in the tire tread with head facing downward. If the tread does not reach the top of Lincoln's head, there is insufficient tread depth)

Spare tire (inflated), jack, lug wrench

Front	Rear

Pass	Fail

LIGHTS

Head lights Tail Lights Brake lights Turn Signals Backup lights

Four-way Flashers

License Plate Light

Both high and low beams operational, cracked, condensation, secured

Lenses intact, tail light working when turned on (red)

Lenses intact, brake light working when brake is applied (red)

Lenses intact, left and right turn signals blink (red lights in rear and yellow lights in front)

Lenses intact, left and right backup lights work (White Light)

Lenses intact, left and right turn signals flash/blink at the same time

Lenses intact, does light stay on

Left	Right

Left	Right

Left	Right

Front	Rear

Left	Right

Left	Right

Front	Rear

Left	Right

Pass	Fail

WINDSHIELD & WINDOWS & WIPERS

Windshield Rear Window Windows

Window controls Windshield wipers

Not cracked, broken or scratched to the degree that impairs vision

Not cracked, broken or scratched to the degree that impairs vision

Windows go up and down, scratched or tinted to the degree that impairs vision

Check handles, push electric buttons

Both wipers are installed on vehicle, windshield wipers work, blades show signs of wear

Pass	Fail

Pass	Fail

Pass	Fail

Front	Rear

Pass	Fail

MIRROR

Mirror Outside

Mirror Inside

Missing, cracked, adjustable Missing, cracked, adjustable

Left	Right

Pass	Fail

BUMPERS

Bumper Front Bumper Rear

Missing, loose, broken

Missing, loose, broken, bent in any way to cause a hazard

Pass	Fail

Pass	Fail

BRAKES

Brakes Emergency Brake

Foot pedal cannot travel more than half way to floor, does brake light stay on

Properly adjusted, check emergency brake by: pull/push emergency brake, apply foot to brake, gently press gas pedal, ensure brake holds vehicle

Pass	Fail

Pass	Fail

Interior

Horn

Defroster Front

Does it work

Ensure hot air blows out above the dash

Pass	Fail

Pass	Fail

Inspector's

Name: 	Signature

Operator

Name: 	Signature

Platoon Sergeant/Platoon Leaders

approval

Date inspection was conducted

Date follow-up inspection was conducted 	 Leave/Pass/Holiday

Defroster Rear

Emergency equipment

Heater

SEATBELTS

Seatbelt Front/Rear (Include shoulder harness during inspection, may have a center seat belt)

Check light on dash, if in the winter ensure it works by allowing the rear windshield to clear up

(OPTIONAL) First aid kit, warning triangle, flashlight, fire extinguisher, blanket, flares, shovel, chains, tools, etc.

Ensure heater works

Missing, frayed, does not snap

Pass	Fail

Pass	Fail

Pass	Fail

Front	Rear

LICENSE/DECALS/INSURANCE

State Drivers License

License Plate/ Inspection Decal

Insurance

Expired, missing

Expired, check sticker/decal to ensure plate is current

Does the operator have valid insurance

Pass	Fail

Pass	Fail

Pass	Fail

UNDER THE HOOD FLUIDS

Brake

Windshield washer Oil

Power Steering

HOSES

BATTERY

Filled to appropriate level Windshield washer fluid Filled to appropriate level Filled to appropriate level

Cuts, cracks, leaks, bulges, chaffing, deterioration

Terminals, clean and tight, held down securely

Pass	Fail

Pass	Fail

Pass	Fail

Pass	Fail

Pass	Fail

Pass	Fail